Davis Southern

Wildman (Yegua) Prospect

BSM Wildman - #1 well

Post Drill Report

January 3, 2019

To: Partners in the BSM Wildman #1

Re:   Davis Southern Operating

BSM Wildman # 1

Polk Co., Texas

Well Log Evaluation

Gentlemen,

We logged and evaluated the test well this morning Jan. 3, 2019 on the Wildman Prospect. The well was drilled to Total Depth of 5500’. The open hole section was logged up to the 9 5/8" surface casing at 1594'. Halliburton ran the Triple Combo Log. No sidewall cores or Formation Tests were run. The well bore evaluation follows.

The well is an oil discovery in the normal pressured Patricia Ray Yegua 1-A Sand. The Patricia Ray Channel sand is 102’ thick. There is a solid 4’ to 5’ feet ( 4864’ to 4869’) of excellent pay 1.0 to 1.3 Ohms of resistivity on top of the sand. Porosity is a clean 32 %. The pay total is a possible 7’-8’ of pay with 0.7 Ohms of resistivity in the bottom of the zone ( 4869’ to 4872’). This zone would be a good 75 to 100 BOPD completion with a good primary cement job. The reservoir is virgin pressure because it is in a separate sand lobe from the Unit Wildman 1 & 2 wells. The amplitude is approximately 35 -40 acres.

The question of whether or not to complete this well is ‘ does this well have the capability to produce 100,000 BO or more making a profitable completed well?’ The answer is definitely yes – but I cannot know how much it will produce. 50,000 BO seems very likely and 100,000 BO + is quite possible with a virgin 35-40 acre reservoir.

Economics are different for each partner, but I would recommend a completion because it is a clean oil zone that is virgin pressure and 35-40 acres in size. The potential reserves are at least 50,000 and an upside of 100,000 BO or more.

An election letter is forthcoming on the completion of this well.

Sincerely,

Patrick A. Kelleher, Geologist