# GEOLOGICAL PROGNOSIS

## GENERAL INFORMATION

| Well Operator: | KEBO Oil & Gas, Inc. | Date Submitted: | 6/1/2020 |
| --- | --- | --- | --- |
| Well Name & No.: | Black Stone #1 (Rattlesnake Mound Prospect) | Field Area: | A.Swingle, A-369 |
| Location Description : |  | Well Type: Directional Max Angle 16.7 degrees | Intermediate + Drilling liner Frio Sand Test |
| County: | Brazoria County | Surface Location (X): | 3,234,548 |
| State: | Texas |  |  |
| Surface Elevation : | 3.0 | Surface Location (Y): | 521,696 |
| T.D. (M.D. / TVDSS) | 11,986.41’ MD / -11,700’ TVD |  |  |
| Type of Completion: | Oil /Gas | Bottom Hole Location (X): | 3,232,630 |
| Surface Casing Seat: | 2,000’ TVD (13 3/8” Casing) |  |  |
| Intermediate Casing Seat: Drilling Liner seat: | 8,000’ TVD (9 5/8” Casing) 10,000 TVD (7 5/8” Casing) | Bottom Hole Location (Y): | 521,297 |

## GEOLOGIC INFORMATION

| Formation Targets: | #### Objective Formation | Primary (P) Secondary (S) | Expected Depth (Est. MD) | Target Radius | Target (X) | Target (Y) | Expected BHP (psi) | Expected Temp. (F) | Expected Production |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | #### “Big Gas” Sand (Frio A) | P | 11,046.41’ | 250’ | 3,232,630 | 521,297 | 9600 # | 227 | Oil/ gas |
|  | #### Grubbs Sand | S | 11,586.41’ | 250’ | 3,232,630 | 521,297 | 10,000 # | 238 | Oil/gas |

### LOGGING PROGRAM

| #### E- Logging Company: | BAKER HUGHES (REP: JIM HANKA (800) 852-9482 | Mud Logging Company: | Stratagragh |
| --- | --- | --- | --- |
| Surface Casing to Int. Casing: | Gamma/Induction | Depth Required: | 9,000-TD |
| Intermediate Casing to Liner: Drilling Liner to TD: | Gamma/Induction Triple Combo (Eclips) | Paleontologist: | N/A |
| Specialty Testing: | Gamma Ray LWD run from Intermediate Casing to Drilling Liner seat. | Depth Required: | N/A |
| Coring: FT Testing: | No Cores No FT | Other: |  |

GENERAL REMARKS

BHPs assume from offset wells – Location approximate and subject to change prior to proposal. With clays known to be present in all objective zones we prefer to use Oil base mud after setting Intermediate Casing.

PROPOSED & OFFSET WELL FORMATIONS

|  | ###### Proposed Well |  | Expected Pressure | ###### Offset #1 |  | Offset #2 |  | Offset #3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Well Operator: | KEBO Oil & Gas |  |  | Zenergy, Inc. |  | Corum Production Company |  | Humble Oil & Refining |  |
| Well Name & No.: | Black Stone #1 |  |  | Roca-Negra #1 |  | Blackstone #1 |  | Houston Farms #3 |  |
| Distance to Proposed Well: | 0’ |  |  | 2067’ MD |  | 4038’ MD |  | 4819’ MD |  |
| Surface Casing | 2000’ MD |  |  | 2007’ MD |  | 2020’ MD |  | 2698’ MD |  |
| Int. Casing/Mud Wt. Drilling Liner/Mud Wt. | 8,235’ MD / 12#/g 10,286 MD/15#/g |  |  | 7,765’ MD / 10.7#/g |  | 7,947’ MD / 10.9#/g |  | 9,883 MD / UNK |  |
| Total Depth/Mud Wt./fl-loss | 11,986.41’ MD/ 17#/g |  |  | 11,600’ / 17.1 # / UNK |  | 11,380’ / 16.6#/ 4.0 cc |  | 11,679’ / UNK/ UNK |  |
| Formation/Fault/Casing Pt. | MD/ TVD | Approx. BHP |  | MD/ TVD | Approx. BHP | MD/ TVD | Approx. BHP | MD/ TVD | Approx. BHP |
| Zenergy Fault (650’) | 10,550’/ -10,268’ | (+/-) 9,500# |  | 10,480’/ -10,458’ | (+/-) 9,500# | 10,780’/ -10,439’ | (+/-) 9,500# | 10,160’/ -10,157’ | (+/-) 9,500# |
| “Big Gas” Sand (Frio A) | 11,046’/ -10,760’ | (+/-) 9,600# |  | 10,855’/ -10,833 | (+/-) 9,600# | 11,230’/ -10,889’ | (+/-) 9,600# | 10,990’/ -10.987’ | (+/-) 9,600# |
| Grubbs Sand | 11,586’/ -11,300’ | (+/-) 10,000# |  | NDE | (+/-) 10,000# | NDE | (+/-) 10,000# | 11,500’/ -11,497’ | (+/-) 10,000# |

CONTACT INFORMATION

| Engineer Responsible for Project: | #### Ken Boester | Office Phone: Cellular Phone: Home Phone: | (361) 643-8821 |
| --- | --- | --- | --- |
|  |  |  | (210) 218-1781 |
| Geologist Responsible for Project: | Paul E Habermas / Rusty Winn / Bob Barnhill | Office Phone / Office Fax: Home Phone / Home Fax: Cellular: | (713) 652-4040 |
|  |  |  | (281) 433-5956 |
| Partner Contact: Partner Information: |  | Office Phone: Fax Phone: Office Phone: |  |