# GEOLOGICAL PROGNOSIS

## GENERAL INFORMATION

| Well Operator: | KEBO Oil & Gas, Inc. | Date Submitted: | 6/1/2021 |
| --- | --- | --- | --- |
| Well Name & No.: | DIAMONDBACK #1 (Diamondback Prospect) | Field Area: | Martin Ranch/ Pegasus |
| Location Description : | J.F. Perry & E. Austin, A-106 | Well Type: Directional Max Angle 30 degrees | Intermediate + Drilling liner Frio Sand Test |
| County: | Brazoria County | Surface Location (X): | 3,225,241.60 |
| State: | Texas |  |  |
| Surface Elevation : | 4.0 | Surface Location (Y): | 521,615.00 |
| T.D. (M.D. / TVDSS) | 12,083.4’ MD / -11,569’ TVD |  |  |
| Type of Completion: | Oil /Gas | Bottom Hole Location (X): | 3,227,178.65 |
| Surface Casing Seat: | 2,000’ MD/TVD (13 3/8” Casing) |  |  |
| Intermediate Casing Seat: Drilling Liner seat: | 8,543’ MD/8,500’ TVD (9 5/8” Casing) 10,300 MD /10,024 TVD (7 5/8” Casing) | Bottom Hole Location (Y): | 520,940.52 |

## GEOLOGIC INFORMATION

| Formation Targets: | #### Objective Formation | Primary (P) Secondary (S) | Expected Depth (Est. MD) | Target Radius | Target (X) | Target (Y) | Expected BHP (psi) | Expected Temp. (F) | Expected Production |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | #### “Big Gas” Sand (Frio A) | P | 10,900.00’ | 50’ | 3,226,623.00 | 521,134.00 | 9600 # | 227 | Oil/ gas |
|  | #### Grubbs Sand | S | 11,600.00’ | 50’ | 3,226,928.36 | 521,027.67 | 10,000 # | 238 | Oil/gas |

### LOGGING PROGRAM

| #### E- Logging Company: | BAKER HUGHES (REP: JIM HANKA (800) 852-9482 | Mud Logging Company: | Stratagragh |
| --- | --- | --- | --- |
| Surface Casing to Int. Casing: | Gamma/Induction Triple Combo (Eclips) | Depth Required: | 8,500-TD |
| Intermediate Casing to Liner: Drilling Liner to TD: | Gamma/Induction Triple Combo (Eclips) | Paleontologist: | N/A |
| Specialty Testing: |  | Depth Required: | N/A |
| Coring: FT Testing: | No Cores No FT | Other: |  |

GENERAL REMARKS

BHPs assume from offset wells – Location approximate and subject to change prior to proposal. With clays known to be present in all objective zones we prefer to use Oil base mud after setting Intermediate Casing.

PROPOSED & OFFSET WELL FORMATIONS

|  | ###### Proposed Well |  | Expected Pressure | ###### Offset #1 |  | Offset #2 |  | Offset #3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Well Operator: | KEBO Oil & Gas |  |  | Stanolind Oil & Gas |  | Humble Oil & Refining |  | KEBO Oil & Gas |  |
| Well Name & No.: | Diamondback #1 |  |  | Houston Farms #1D |  | Houston Farms #4 |  | Blackstone #1 |  |
| Distance to Proposed Well: | 0’ |  |  | 1620’ |  | 1877’ |  | 5883’ |  |
| Surface Casing | 2000’ MD |  |  | 2516’ MD |  | 2020’ MD |  | 2698’ MD |  |
| Int. Casing/Mud Wt. Drilling Liner/Mud Wt. | 8,543.62’ MD / 12#/g 10,300 MD/15#/g |  |  | 9,420’ MD / 13.4#/g |  | 9,893’ MD / UNK |  | 7,893 MD / 11.0/g /4.0cc |  |
| Total Depth/Mud Wt./fl-loss | 12,083’ MD/ 17#/g |  |  | 12,836 MD / 16.8 #/g |  | 11,506’ / 16.0#/ g |  | 12,000’ / 16.9#/g/ UNK |  |
| Formation/Fault/Casing Pt. | MD/ TVD | Approx. BHP |  | MD/ TVD | Approx. BHP | MD/ TVD | Approx. BHP | MD/ TVD | Approx. BHP |
| Red/ Pink Fault (700’) | 9820’/ 9618 | (+/-) 9,000# |  | NA |  | NDE |  | NA |  |
| Zenergy Fault (650’) | 9,100’/ 8,897’ | (+/-) 9,500# |  | 8,845’/ 8,845’ | (+/-) 9,500# | 10,500’/ 10,500’ | (+/-) 9,500# | 10,930’/ 10,650’ | (+/-) 9,500# |
| “Big Gas” Sand (Frio A) | 10,900’/ 10,550’ | (+/-) 9,600# |  | 10,630’/ 10,630 | (+/-) 9,600# | 11,367’/ 11,367 | (+/-) 9,600# | 11,150’/ 10865’ | (+/-) 9,600# |
| Grubbs Sand | 11,600’/ 11,110’ | (+/-) 10,000# |  | 11,180/ 11,180 | (+/-) 10,000# | NDE | (+/-) 10,000# | 11,608’/ 11,320’ | (+/-) 10,000# |

CONTACT INFORMATION

| Engineer Responsible for Project: | #### Ken Boester | Office Phone: Cellular Phone: Home Phone: | (361) 643-8821 |
| --- | --- | --- | --- |
|  |  |  | (210) 218-1781 |
| Geologist Responsible for Project: | Paul E Habermas / Rusty Winn / Bob Barnhill | Office Phone / Office Fax: Home Phone / Home Fax: Cellular: | (713) 652-4040 |
|  |  |  | (281) 433-5956 |
| Partner Contact: Partner Information: |  | Office Phone: Fax Phone: Office Phone: |  |